# SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifiers

Product name	: 2-Phenoxyethanol

Product Number	: 77699

Brand	: Sigma-Aldrich

Index-No.	: 603-098-00-9

CAS-No.	: 122-99-6

# Relevant identified uses of the substance or mixture and uses advised against

Identified uses	: Laboratory chemicals, Manufacture of substances

# Details of the supplier of the safety data sheet

Company	              :  Swadesh Life Science

H-103, Sumel 7, Near Soni ni chali

BRTS, Rakhiyal,

Ahmedabad,Gujarat, India

# SECTION 2: Hazards identification

  - Classification of the substance or mixture

# Classification according to Regulation (EC) No 1272/2008

Acute toxicity, Oral (Category 4), H302 Eye irritation (Category 2), H319

For the full text of the H-Statements mentioned in this Section, see Section 16.

# Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram

Signal word	Warning Hazard statement(s)

H302	Harmful if swallowed.

H319	Causes serious eye irritation.

Precautionary statement(s)

P264	Wash skin thoroughly after handling.

P270	Do not eat, drink or smoke when using this product.

P280	Wear eye protection/ face protection.

P301 + P312	IF SWALLOWED: Call a POISON CENTER/ doctor if you feel

unwell.

P305 + P351 + P338	IF IN EYES: Rinse cautiously with water for several minutes.

Remove contact lenses, if present and easy to do. Continue rinsing.

P337 + P313	If eye irritation persists: Get medical advice/ attention.

Supplemental Hazard Statements

none

# Reduced Labeling (<= 125 ml)

Pictogram

Signal word	Warning

Hazard statement(s)	none

Precautionary statement(s)

Supplemental Hazard Statements

none none

# Reduced Labeling (<= 125 ml)

Pictogram

Signal word	Warning

Hazard statement(s)	none

Precautionary statement(s)

Supplemental Hazard Statements

# Other hazards

none none

This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.

# SECTION 3: Composition/information on ingredients

3.1	Substances

Synonyms	: Ethylene glycol monophenyl ether Phenylglycol

Formula	: C8H10O2

Molecular weight	: 138,16 g/mol

CAS-No.	: 122-99-6

EC-No.	: 204-589-7

Index-No.	: 603-098-00-9

| Component | Classification | Concentration |
| --- | --- | --- |
| 2-Phenoxyethanol |  |  |
| CAS-No. 122-99-6 EC-No. 204-589-7 Index-No. 603-098-00-9 | Acute Tox. 4; Eye Irrit. 2; H302, H319 | <= 100 % |

# SECTION 4: First aid measures

  - Description of first-aid measures General advice

Show this material safety data sheet to the doctor in attendance.

# If inhaled

After inhalation: fresh air.

# In case of skin contact

In case of skin contact: Take off immediately all contaminated clothing. Rinse skin with water/ shower.

# In case of eye contact

After eye contact: rinse out with plenty of water. Call in ophthalmologist. Remove contact lenses.

# If swallowed

After swallowing: immediately make victim drink water (two glasses at most). Consult a physician.

# Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

# Indication of any immediate medical attention and special treatment needed

No data available

# SECTION 5: Firefighting measures

  - Extinguishing media Suitable extinguishing media

Water Foam Carbon dioxide (CO2) Dry powder

# Unsuitable extinguishing media

For this substance/mixture no limitations of extinguishing agents are given.

# Special hazards arising from the substance or mixture

Carbon oxides Combustible.

Vapors are heavier than air and may spread along floors. Forms explosive mixtures with air on intense heating.

Development of hazardous combustion gases or vapours possible in the event of fire.

# Advice for firefighters

In the event of fire, wear self-contained breathing apparatus.

# Further information

Prevent fire extinguishing water from contaminating surface water or the ground water system.

# SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

Advice for non-emergency personnel: Do not breathe vapors, aerosols. Avoid substance contact. Ensure adequate ventilation. Evacuate the danger area, observe emergency procedures, consult an expert.

For personal protection see section 8.

# Environmental precautions

Do not let product enter drains.

# Methods and materials for containment and cleaning up

Cover drains. Collect, bind, and pump off spills. Observe possible material restrictions (see sections 7 and 10). Take up with liquid-absorbent material (e.g. Chemizorb® ). Dispose of properly. Clean up affected area.

# Reference to other sections

For disposal see section 13.

# SECTION 7: Handling and storage

  - Precautions for safe handling

For precautions see section 2.2.

# Conditions for safe storage, including any incompatibilities Storage conditions

Tightly closed.

Store under inert gas.

# Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

# SECTION 8: Exposure controls/personal protection

  - Control parameters

# Ingredients with workplace control parameters

  - Exposure controls

# Personal protective equipment Eye/face protection

Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU). Safety glasses

# Skin protection

This recommendation applies only to the product stated in the safety data sheet, supplied by us and for the designated use. When dissolving in or mixing with other substances and under conditions deviating from those stated in EN374 please contact the supplier of CE-approved gloves (e.g. KCL GmbH, D-36124 Eichenzell, Internet: www.kcl.de).

Full contact

Material: Chloroprene

Minimum layer thickness: 0,65 mm Break through time: 480 min Material tested:KCL 720 Camapren®

This recommendation applies only to the product stated in the safety data sheet, supplied by us and for the designated use. When dissolving in or mixing with other substances and under conditions deviating from those stated in EN374 please contact the supplier of CE-approved gloves (e.g. KCL GmbH, D-36124 Eichenzell, Internet: www.kcl.de).

Splash contact Material: Latex gloves

Minimum layer thickness: 0,6 mm Break through time: 30 min

Material tested:Lapren® (KCL 706 / Aldrich Z677558, Size M)

# Body Protection

protective clothing

# Respiratory protection

Recommended Filter type: Filter A (acc. to DIN 3181) for vapours of organic compounds

The entrepeneur has to ensure that maintenance, cleaning and testing of respiratory protective devices are carried out according to the instructions of the producer.

These measures have to be properly documented.

# SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

    - Appearance	Form: liquid Color: colorless

    - Odor	weak

    - Odor Threshold	No data available

    - pH	7 at 10 g/l at 23 °C

    - Melting

point/freezing point

    - Initial boiling point and boiling range

    - Flash point                      	126 °C - closed cup

    - Evaporation rate	No data available

Melting point/range: 11 - 13 °C

244 - 246 °C

    - Flammability (solid, gas)

    - Upper/lower flammability or explosive limits

No data available

Upper explosion limit: 9 %(V) Lower explosion limit: 1,4 %(V)

    - Vapor pressure	0,02 hPa at 25 °C - OECD Test Guideline 104

0,01 hPa at 20 °C - OECD Test Guideline 104

    - Vapor density	4,77 - (Air = 1.0)

    - Relative density	1,11 at 20 °C - OECD Test Guideline 109

    - Water solubility	28,6 g/l at 20,7 °C - OECD Test Guideline 105

    - Partition coefficient: n-octanol/water

    - Autoignition temperature

    - Decomposition temperature

log Pow: 1,107 - OECD Test Guideline 117 - Bioaccumulation is not expected.

475 °C

at > 997 - < 1.001 hPa - DIN 51794

No data available

    - Viscosity	Viscosity, kinematic: No data available

Viscosity, dynamic: 41 mPa.s at 19,8 °C - OECD Test Guideline 11419 mPa.s at 40,5 °C - OECD Test Guideline 114

    - Explosive properties	No data available

    - Oxidizing properties	No data available

# Other safety information

Surface tension	70,7 mN/m at 1g/l at 19,9 °C

- OECD Test Guideline 115

Relative vapor density

4,77 - (Air = 1.0)

# SECTION 10: Stability and reactivity

  - Reactivity

Forms explosive mixtures with air on intense heating.

A range from approx. 15 Kelvin below the flash point is to be rated as critical.

# Chemical stability

The product is chemically stable under standard ambient conditions (room temperature) .

# Possibility of hazardous reactions

No data available

# Conditions to avoid

Strong heating.

# Incompatible materials

Strong oxidizing agents

# Hazardous decomposition products

In the event of fire: see section 5

# SECTION 11: Toxicological information

  - Information on toxicological effects Acute toxicity

LD50 Oral - Rat - male and female - 1.850 mg/kg (OECD Test Guideline 401)

LC50 Inhalation - Rat - male and female - 6 h - > 1.000 mg/m3 (OECD Test Guideline 412)

# Skin corrosion/irritation

Skin - Rabbit

Result: No skin irritation - 4 h (OECD Test Guideline 404)

# Serious eye damage/eye irritation

Eyes - Rabbit

Result: Causes serious eye irritation. - 15 Days (OECD Test Guideline 405)

Respiratory or skin sensitization Maximization Test - Guinea pig Result: negative

(OECD Test Guideline 406)

# Germ cell mutagenicity

Ames test

Escherichia coli/Salmonella typhimurium Result: negative

In vitro mammalian cell gene mutation test Chinese hamster lung cells

Result: Positive results were obtained in some in vitro tests. Mutagenicity (mammal cell test): chromosome aberration. Chinese hamster lung cells

Result: negative

OECD Test Guideline 474

Mouse - male - Red blood cells (erythrocytes) Result: negative

OECD Test Guideline 486 Rat - male - Liver cells Result: negative

Rat - male and female - Bone marrow Result: negative

Remarks:

(ECHA)

# Carcinogenicity

IARC:	No ingredient of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

# Reproductive toxicity

No data available

# Specific target organ toxicity - single exposure

No data available

# Specific target organ toxicity - repeated exposure

No data available

# Aspiration hazard

No data available

# Additional Information

Repeated dose toxicity - Rat - male and female - Oral - 13 Weeks - NOAEL (No observed adverse effect level) - 369 mg/kg

Repeated dose toxicity - Rabbit - male and female - Dermal - 13 Weeks - NOAEL (No observed adverse effect level) - 500 mg/kg - LOAEL (Lowest observed adverse effect level)

- > 500 mg/kg RTECS: KM0350000

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

# SECTION 12: Ecological information

  - Toxicity

Toxicity to fish	static test LC50 - Leuciscus idus (Golden orfe) - > 220 - < 460 mg/l

- 96 h

(DIN 38412)

Toxicity to daphnia and other aquatic invertebrates

static test EC50 - Daphnia magna (Water flea) - > 500 mg/l - 48 h (OECD Test Guideline 202)

Toxicity to algae	static test ErC50 - Desmodesmus subspicatus (green algae) - 625 mg/l - 72 h

(Regulation (EC) No. 440/2008, Annex, C.3)

# Persistence and degradability

Biodegradability	aerobic - Exposure time 3 d

Result: 98 % - Readily biodegradable. (OECD Test Guideline 301A)

Chemical Oxygen Demand (COD)

2,127 mg/g Remarks: (IUCLID)

# Bioaccumulative potential

No data available

# Mobility in soil

No data available

# Results of PBT and vPvB assessment

This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.